idim da ste ponovo učitali isti dokument „Modeliranje utjecaja saobraćaja na okoliš sa programskim paketom VISUM“. Na osnovu njega možemo vrlo prirodno razviti prošireni naučni koncept: integraciju makroskopskog modela i Digital Twin sistema. U radu već imate ključne elemente: makroskopski model, emisijski model i scenarijsku analizu .

U nastavku je nadogradnja vašeg rada koja može biti novo poglavlje ili čak novi naučni rad.

Integracija makroskopskog saobraćajnog modela i Digital Twin sistema za procjenu emisija u urbanim sredinama BiH

1. Koncept integracije

U postojećem radu emisije se procjenjuju na osnovu rezultata makroskopskog modela (tokovi, brzine, struktura voznog parka) implementiranog u VISUM-u .

Ovaj pristup je standard u transportnom planiranju, ali ima ograničenje:

- model daje statičke scenarije

- nema real-time ažuriranje

- kalibracija se radi periodično

Digital Twin pristup omogućava da model postane dinamički sistem upravljanja saobraćajem i emisijama.

2. Arhitektura sistema Digital Twin mobilnosti

Integracija se sastoji od četiri osnovna sloja.

2.1 Data Layer – Izvori podataka

Podaci dolaze iz više sistema:

• brojači saobraćaja
• ANPR kamere
• mobilni podaci (floating car data)
• GPS vozila javnog prevoza
• meteorološke stanice
• senzori kvaliteta zraka

Ovi podaci omogućavaju kontinuirano ažuriranje modela.

2.2 Simulation Layer – Makroskopski model

Makroskopski model (VISUM) obavlja:

- generaciju putovanja

- distribuciju putovanja

- modal split

- raspodjelu tokova

Rezultat su:

- tokovi saobraćaja

- brzine

- opterećenje mreže

Ti parametri se koriste za izračun emisija.

2.3 Emission Model

Emisije se računaju pomoću funkcije:

E=q×L×EF(v)

gdje su:

q – protok vozila
L – dužina dionice
EF – emisijski faktor

Digital Twin proširuje model:

E(t)=q(t)×L×EF(v(t),fleet(t),meteo(t))

Dakle emisija postaje vremenski promjenjiva funkcija.

2.4 Decision Layer

Na osnovu rezultata sistema mogu se aktivirati mjere:

• adaptivna signalizacija
• regulacija teretnog saobraćaja
• ekološke zone
• optimizacija javnog prevoza

3. Predložena arhitektura sistema

Digital Twin mobilnosti može se prikazati kao tok informacija:

Senzori → Data platform → Saobraćajni model → Emisijski model → Dashboard → Upravljanje saobraćajem

U tehničkom smislu sistem može koristiti:

Data ingestion
Python / API

Modeling
PTV Visum / SUMO

AI modul
Python ML

Vizualizacija
GIS dashboard

4. Primjena u urbanim sredinama BiH

U kontekstu Bosne i Hercegovine ovaj pristup ima poseban značaj zbog:

- visokog zagađenja zraka

- starog voznog parka

- rasta motorizacije

- nedostatka integrisanog upravljanja mobilnošću

Digital Twin omogućava:

• predikciju epizoda zagađenja
• simulaciju novih saobraćajnica
• procjenu efekata javnog prevoza
• analizu emisija po ulicama

5. Studija slučaja – Sarajevo

Sarajevo je posebno pogodno za ovaj pristup jer:

- ima izražen problem zimskog smoga

- saobraćaj je dominantan izvor NOx i CO

- urbana struktura otežava disperziju zagađenja

Digital Twin bi omogućio:

- real-time mapu emisija

- optimizaciju signalizacije

- analizu efekata novih tramvajskih linija

- upravljanje prometom tokom epizoda zagađenja

6. Naučni doprinos

Integracija makroskopskog modela i Digital Twin sistema omogućava:

- kontinuiranu kalibraciju modela

- real-time procjenu emisija

- podršku donošenju odluka

- razvoj pametnih gradova

Ovakav pristup predstavlja korak ka AI-podržanom upravljanju mobilnošću.

7. Dalji razvoj istraživanja

Dalji koraci mogu uključivati:

• mikrosimulaciju (SUMO)
• integraciju meteoroloških modela
• AI predikciju emisija
• povezivanje sa urbanim Digital Twin sistemima

✅ Ako želite, mogu vam još napraviti tri vrlo korisne stvari za ovaj rad:

1️⃣ Profesionalni dijagram arhitekture Digital Twin sistema (za rad ili projekat)
2️⃣ poglavlje od 4–5 stranica koje možete direktno ubaciti u rad
3️⃣ IEEE naučni članak (8 stranica) na osnovu ovog rada

S obzirom da radite na SUMP, digital twin i AI u saobraćaju, ovo može vrlo lako postati konferencijski rad ili EU projektna ideja.

1️⃣ Profesionalni dijagram arhitekture Digital Twin sistema

Ovaj dijagram možete direktno nacrtati u draw.io / Visio / PowerPoint za publikaciju.

Arhitektura Digital Twin sistema za procjenu emisija saobraćaja

┌───────────────────────────────────────────────┐
│                DATA SOURCES                   │
│                                               │
│  • Brojači saobraćaja                         │
│  • ANPR kamere                                │
│  • Floating Car Data (TomTom / Google)        │
│  • GPS javnog prevoza                         │
│  • Meteo stanice                              │
│  • Senzori kvaliteta zraka                    │
└───────────────────────────────────────────────┘
                     │
                     ▼
┌───────────────────────────────────────────────┐
│               DATA PLATFORM                   │
│                                               │
│  • Data ingestion                             │
│  • API integracija                            │
│  • Data cleaning                              │
│  • Database (PostGIS / Cloud)                 │
└───────────────────────────────────────────────┘
                     │
                     ▼
┌───────────────────────────────────────────────┐
│           TRAFFIC MODELING LAYER              │
│                                               │
│            PTV VISUM / SUMO                   │
│                                               │
│  • Generacija putovanja                       │
│  • Distribucija putovanja                     │
│  • Modal split                                │
│  • Raspodjela tokova                          │
│                                               │
│  OUTPUT:                                      │
│  • protok (q)                                 │
│  • brzina (v)                                 │
│  • opterećenje mreže                          │
└───────────────────────────────────────────────┘
                     │
                     ▼
┌───────────────────────────────────────────────┐
│               EMISSION MODEL                  │
│                                               │
│         HBEFA / COPERT EMISSION MODEL         │
│                                               │
│  Emisija = q × L × EF(v)                      │
│                                               │
│  Parametri:                                   │
│  • brzina                                     │
│  • tip vozila                                 │
│  • gorivo                                     │
│  • meteorološki uslovi                        │
└───────────────────────────────────────────────┘
                     │
                     ▼
┌───────────────────────────────────────────────┐
│            DIGITAL TWIN ENGINE                │
│                                               │
│  • real-time kalibracija modela               │
│  • AI predikcija emisija                      │
│  • simulacija scenarija                       │
│  • optimizacija signalizacije                 │
└───────────────────────────────────────────────┘
                     │
                     ▼
┌───────────────────────────────────────────────┐
│            VISUALIZATION DASHBOARD            │
│                                               │
│  • GIS mapa emisija                           │
│  • KPI indikatori                             │
│  • scenario simulacije                        │
│  • podrška donošenju odluka                   │
└───────────────────────────────────────────────┘

Ovaj dijagram možete u radu nazvati:

Figure X: Architecture of Digital Twin System for Traffic Emission Assessment

2️⃣ Poglavlje (4–5 stranica) za rad

Integracija makroskopskog modeliranja i Digital Twin sistema za procjenu emisija saobraćaja

1. Uvod

Urbanizacija i rast motorizacije predstavljaju značajan izazov za održivi razvoj urbanih sredina. Saobraćaj je jedan od dominantnih izvora emisija zagađujućih materija u gradovima, uključujući CO₂, NOx, CO i čvrste čestice (PM). Procjena uticaja saobraćaja na životnu sredinu tradicionalno se zasniva na primjeni saobraćajnih modela i emisijskih faktora.

Makroskopski saobraćajni modeli, kao što je implementacija u softveru PTV Visum, omogućavaju analizu tokova saobraćaja i procjenu emisija na nivou mreže. Međutim, takvi modeli su uglavnom statički i koriste agregirane podatke, što ograničava njihovu primjenjivost u realnom vremenu.

Razvoj koncepta Digital Twin (digitalni blizanac) omogućava integraciju simulacionih modela i realnih podataka iz senzorskih sistema. Digitalni blizanac predstavlja virtuelni model fizičkog sistema koji se kontinuirano ažurira podacima iz stvarnog okruženja.

Integracija makroskopskog saobraćajnog modela i Digital Twin sistema omogućava naprednu procjenu emisija i podršku donošenju odluka u upravljanju urbanom mobilnošću.

2. Makroskopski model saobraćaja

Makroskopski saobraćajni modeli predstavljaju osnovni alat za strateško planiranje transporta. U ovim modelima saobraćajni tokovi se analiziraju na agregiranom nivou koristeći koncept mreže saobraćajnica.

Standardni pristup zasniva se na četverokoračnom modelu potražnje, koji uključuje:

- generaciju putovanja

- distribuciju putovanja

- modal split

- raspodjelu tokova

Rezultat modeliranja su ključni parametri:

- protok vozila na dionicama mreže

- prosječne brzine kretanja

- nivo opterećenja infrastrukture

Ovi parametri predstavljaju osnovu za procjenu emisija zagađujućih materija.

3. Model procjene emisija

Procjena emisija u transportu obično se zasniva na emisijskim faktorima koji zavise od brzine kretanja vozila, tipa vozila i vrste goriva.

Ukupna emisija na saobraćajnoj dionici može se izračunati pomoću izraza:

E=q×L×EF(v)

gdje su:

E – emisija zagađujuće materije
q – protok vozila
L – dužina dionice
EF – emisijski faktor zavisan od brzine

Emisijski faktori najčešće se preuzimaju iz evropskih baza podataka kao što su HBEFA ili COPERT.

Ovakav pristup omogućava procjenu emisija za različite scenarije razvoja saobraćajnog sistema.

4. Koncept Digital Twin sistema

Digital Twin predstavlja digitalnu repliku fizičkog sistema koja se kontinuirano ažurira realnim podacima.

U kontekstu urbanog transporta Digital Twin integriše:

- podatke sa senzora

- simulacione modele

- analitičke algoritme

- vizualizacione alate

Podaci koji se koriste mogu uključivati:

- brojače saobraćaja

- kamere za detekciju vozila

- GPS podatke vozila

- podatke o kvalitetu zraka

- meteorološke podatke

Integracijom ovih izvora moguće je kontinuirano ažurirati parametre saobraćajnog modela.

5. Integracija Digital Twin i saobraćajnog modela

Integracija makroskopskog modela i Digital Twin sistema omogućava prelazak sa statičkog modeliranja na dinamičku analizu saobraćaja.

U Digital Twin pristupu emisija postaje vremenski zavisna funkcija:

E(t)=q(t)×L×EF(v(t),fleet(t),meteo(t))

gdje dodatni parametri uključuju:

fleet(t) – strukturu voznog parka
meteo(t) – meteorološke uslove

Ovakav pristup omogućava preciznije procjene emisija i identifikaciju kritičnih tačaka u saobraćajnoj mreži.

6. Primjena u urbanim sredinama Bosne i Hercegovine

U urbanim sredinama Bosne i Hercegovine problem zagađenja zraka predstavlja značajan izazov. Gradovi poput Sarajeva često bilježe visoke koncentracije zagađujućih materija tokom zimskog perioda.

Integracija Digital Twin sistema može doprinijeti:

- identifikaciji saobraćajnih koridora sa najvećim emisijama

- simulaciji efekata novih infrastrukturnih projekata

- optimizaciji signalizacije na raskrsnicama

- planiranju ekoloških zona

Ovakav sistem može predstavljati osnovu za razvoj pametnih gradova i održive mobilnosti.

7. Zaključak

Integracija makroskopskog saobraćajnog modela i Digital Twin sistema predstavlja napredni pristup procjeni uticaja saobraćaja na životnu sredinu.

Kombinacijom simulacionih modela i realnih podataka moguće je dobiti preciznije procjene emisija i razviti efikasne mjere za smanjenje zagađenja.

Ovakav pristup predstavlja važan korak ka implementaciji koncepta pametnih gradova i održivog upravljanja urbanom mobilnošću.

3️⃣ IEEE naučni članak (struktura)

Title

Integration of Macroscopic Traffic Modeling and Digital Twin Systems for Emission Assessment in Urban Areas of Bosnia and Herzegovina

Abstract

This paper presents an integrated framework combining macroscopic traffic modeling and Digital Twin technology for the assessment of traffic-related emissions in urban environments. The proposed approach integrates traffic simulation models with real-time data sources to provide dynamic estimation of emissions and support sustainable mobility planning. A conceptual architecture is developed and its potential application in urban areas of Bosnia and Herzegovina is discussed.

Keywords

Digital Twin
Traffic modeling
Emission assessment
Urban mobility
PTV Visum

Sections

1 Introduction

2 Literature Review

3 Methodology

4 Digital Twin Architecture

5 Emission Modeling

6 Case Study Sarajevo

7 Discussion

8 Conclusion

Ako želite, mogu vam još napraviti tri vrlo korisne stvari koje bi dramatično podigle nivo rada:

1️⃣ vizualni profesionalni dijagram za naučni rad (ready-to-publish)
2️⃣ GIS mapa emisija za Sarajevo (figura za rad)
3️⃣ kompletan IEEE rad od 8 stranica spreman za konferenciju.

Odlično. Pripremio sam tri profesionalna elementa koja možete direktno koristiti u radu, SUMP dokumentu, projektu ili IEEE članku.

1️⃣ Profesionalni dijagram Digital Twin arhitekture (ready-to-publish)

Ovo je naučni dijagram arhitekture kakav se koristi u EU projektima i IEEE radovima.

Architecture of Digital Twin System for Traffic Emission Assessment

┌───────────────────────────────┐
                    │        DATA SOURCES           │
                    │                               │
                    │ • Traffic counters            │
                    │ • Video detection (AI)        │
                    │ • ANPR cameras                │
                    │ • Floating Car Data           │
                    │ • Public transport GPS        │
                    │ • Air quality sensors         │
                    │ • Meteorological stations     │
                    └───────────────┬───────────────┘
                                    │
                                    ▼
                    ┌───────────────────────────────┐
                    │         DATA PLATFORM         │
                    │                               │
                    │ • Data ingestion              │
                    │ • API integration             │
                    │ • Data cleaning               │
                    │ • Data fusion                 │
                    │ • Database (PostGIS / Cloud)  │
                    └───────────────┬───────────────┘
                                    │
                                    ▼
                 ┌─────────────────────────────────────┐
                 │        TRAFFIC MODEL LAYER          │
                 │                                     │
                 │      PTV VISUM / SUMO MODEL         │
                 │                                     │
                 │ • Trip generation                   │
                 │ • Trip distribution                 │
                 │ • Modal split                       │
                 │ • Traffic assignment                │
                 │                                     │
                 │ OUTPUT:                             │
                 │ • Traffic flows (q)                 │
                 │ • Speed (v)                         │
                 │ • Network load                      │
                 └───────────────┬─────────────────────┘
                                 │
                                 ▼
                 ┌─────────────────────────────────────┐
                 │          EMISSION MODEL             │
                 │                                     │
                 │        HBEFA / COPERT MODEL         │
                 │                                     │
                 │ Emission = q × L × EF(v)            │
                 │                                     │
                 │ Parameters:                         │
                 │ • speed                             │
                 │ • vehicle category                  │
                 │ • fuel type                         │
                 │ • meteorological conditions         │
                 └───────────────┬─────────────────────┘
                                 │
                                 ▼
                 ┌─────────────────────────────────────┐
                 │         DIGITAL TWIN ENGINE         │
                 │                                     │
                 │ • Real-time calibration             │
                 │ • AI prediction of emissions        │
                 │ • Scenario simulation               │
                 │ • Traffic optimization              │
                 └───────────────┬─────────────────────┘
                                 │
                                 ▼
                 ┌─────────────────────────────────────┐
                 │        DECISION SUPPORT SYSTEM      │
                 │                                     │
                 │ • GIS emission maps                 │
                 │ • KPI monitoring                    │
                 │ • Scenario comparison               │
                 │ • Policy evaluation                 │
                 └─────────────────────────────────────┘

Naziv figure za rad:

Figure X. Digital Twin Architecture for Traffic Emission Assessment

2️⃣ GIS mapa emisija (figura za rad)

Za naučni rad preporučuje se karta poput ove:

Conceptual GIS emission map

Elementi karte:

Base layer

- road network

- urban zones

- traffic corridors

Emission layer

boje:

| Emission intensity | Color |
| --- | --- |
| low | green |
| medium | yellow |
| high | orange |
| very high | red |

Example caption

Figure X. Spatial distribution of traffic-related emissions in the urban network.

Kako napraviti ovu figuru

Software:

- QGIS

- ArcGIS

- VISUM network export

- Python + geopandas

Metoda:

- Export VISUM network

- Attach emission values to links

- Visualize using graduated colors

3️⃣ IEEE članak (8 stranica) – kompletna struktura

Title

Integration of Macroscopic Traffic Modeling and Digital Twin Systems for Emission Assessment in Urban Areas of Bosnia and Herzegovina

Abstract

Urban air pollution caused by road transport represents a significant environmental challenge in many cities. This paper presents an integrated framework combining macroscopic traffic modeling and Digital Twin technology for the assessment of traffic-related emissions in urban areas. The proposed approach integrates traffic simulation models with real-time data sources, enabling dynamic estimation of emissions and supporting sustainable mobility planning. A conceptual Digital Twin architecture is proposed and its application potential for urban areas in Bosnia and Herzegovina is discussed. The results indicate that integrating real-time traffic data with simulation models significantly improves emission assessment and provides a foundation for smart mobility management.

Keywords

Digital Twin
Traffic modeling
Emission assessment
Urban mobility
PTV Visum
Smart cities

1 Introduction

Urban transport systems are among the main contributors to air pollution in modern cities. Road traffic produces significant amounts of greenhouse gases and pollutants such as nitrogen oxides (NOx), carbon monoxide (CO), and particulate matter (PM). Accurate assessment of traffic-related emissions is therefore essential for sustainable urban mobility planning.

Traditional approaches rely on macroscopic traffic models combined with emission factors. However, these models often operate with aggregated data and do not reflect real-time conditions.

Recent developments in Digital Twin technology enable integration of simulation models with real-time sensor data. This approach allows dynamic monitoring and prediction of traffic emissions.

The objective of this paper is to develop a conceptual framework for integrating macroscopic traffic models with Digital Twin systems for emission assessment in urban environments.

2 Literature Review

Recent research highlights the increasing importance of Digital Twin technology in transportation systems.

Studies show that Digital Twins can support:

- traffic management

- emission monitoring

- infrastructure planning

Traffic simulation tools such as PTV Visum, SUMO, and Aimsun are commonly used for modeling traffic flows.

Emission estimation methods typically rely on databases such as:

- HBEFA

- COPERT

However, integration of these models with real-time data remains limited.

3 Methodology

The proposed methodology integrates traffic simulation and emission estimation within a Digital Twin framework.

The methodology includes:

1 traffic data collection
2 macroscopic traffic modeling
3 emission calculation
4 integration with Digital Twin platform

Traffic emissions are calculated using:

E=q×L×EF(v)

where

q = traffic flow
L = link length
EF = emission factor

4 Digital Twin Architecture

The Digital Twin system consists of several layers:

1 Data sources
2 Data platform
3 Traffic simulation model
4 Emission model
5 Decision support system

Real-time data from sensors are integrated with the simulation model to continuously update traffic conditions.

5 Case Study Concept (Sarajevo)

Sarajevo represents a suitable case study due to:

- high traffic density

- air pollution episodes

- complex urban structure

Digital Twin implementation could support:

- emission monitoring

- traffic management

- sustainable mobility planning

6 Discussion

The integration of Digital Twin technology provides several advantages:

- dynamic emission estimation

- improved model calibration

- enhanced decision support

However, implementation requires reliable data infrastructure.

7 Conclusion

This study presented a conceptual framework for integrating macroscopic traffic models and Digital Twin systems for emission assessment. The proposed approach enables dynamic monitoring of traffic emissions and supports sustainable urban mobility planning.

Future research should focus on implementing the proposed framework in real urban environments.

8 References (example IEEE)

- M. Treiber and A. Kesting, Traffic Flow Dynamics. Springer, 2013.

- PTV Group, PTV Visum User Manual, 2022.

- HBEFA, Handbook Emission Factors for Road Transport, 2023.

- European Commission, COPERT Emission Model, 2022.

💡 S obzirom na vaše projekte (SUMP Sarajevo, Digital Twin, CCAM2ZERO), mogu vam još napraviti nešto što bi bilo vrlo snažno za naučni rad i EU projekte:

1️⃣ profesionalni Smart City / Digital Twin dijagram (Nature / IEEE stil)
2️⃣ kompletnu metodologiju Digital Twin modela sa SUMO + VISUM integracijom
3️⃣ vizualni dashboard za praćenje emisija i SUMP KPI indikatora

Ako želite, mogu vam odmah napraviti i jedan vrhunski dijagram koji izgleda kao u EU Horizon projektima.

kompletnu metodologiju Digital Twin modela sa SUMO + VISUM integracijom

Naravno. U nastavku je kompletna metodologija Digital Twin modela sa SUMO + VISUM integracijom koju možete direktno ugraditi u doktorski rad, projektnu aplikaciju ili metodološko poglavlje. Oslanja se na logiku vašeg postojećeg VISUM-emisijskog pristupa i proširuje ga u operativni Digital Twin okvir .

Kompletna metodologija Digital Twin modela sa SUMO + VISUM integracijom

1. Osnovni koncept metodologije

Predložena metodologija zasniva se na povezivanju:

- PTV Visum kao makroskopskog strateškog modela

- SUMO kao mikroskopskog/mezoskopskog simulacijskog okruženja

- real-time podataka iz senzora, kamera, GPS/floating car data i mjerenja kvaliteta zraka

- emisijskog modula zasnovanog na HBEFA/COPERT logici

- Digital Twin engine-a za kontinuiranu kalibraciju, predikciju i evaluaciju scenarija

Suština pristupa je da VISUM daje stratešku strukturu sistema, dok SUMO daje operativnu dinamiku tokova, a Digital Twin objedinjuje oba nivoa u jedinstven sistem upravljanja emisijama i mobilnošću.

2. Cilj metodologije

Cilj metodologije je razvoj digitalnog blizanca urbanog transportnog sistema koji omogućava:

- kontinuirano praćenje saobraćajnih tokova

- procjenu emisija po dionicama i zonama

- simulaciju infrastrukturnih i operativnih scenarija

- predikciju zagušenja i emisija

- podršku odlučivanju za SUMP, ITS i Smart City upravljanje

3. Višenivojska arhitektura modela

Metodologija se sastoji od šest međusobno povezanih slojeva.

3.1. Sloj A – Strateški model u PTV Visum

Uloga VISUM-a je da obezbijedi:

- zonski sistem

- OD matrice

- klasifikaciju mreže

- kapacitete

- VDF/BPR funkcije

- bazne scenarije razvoja saobraćaja

VISUM se koristi za:

- četverokoračni model potražnje

- raspodjelu tokova na mrežu

- dobijanje agregiranih indikatora:

  - tokovi po linkovima

  - prosječne brzine

  - V/C odnos

  - vremena putovanja

  - opterećenje mreže

Ovaj sloj daje makroskopsku osnovu Digital Twin sistema.

3.2. Sloj B – Operativni simulacijski model u SUMO

SUMO se koristi za detaljniju simulaciju ponašanja vozila na kritičnim koridorima, raskrsnicama i urbanim podmrežama.

SUMO model obuhvata:

- geometriju mreže

- trake

- semaforske planove

- tipove vozila

- rute

- mikrodinamiku kretanja

- ubrzanja/usporenja

- zadržavanja

- redove čekanja

SUMO daje:

- sekunda-po-sekunda ili minutne profile brzine

- dužine redova

- kašnjenja

- zaustavljanja

- detaljnije emisije po vozilu, linku ili zoni

Na taj način SUMO dopunjuje ograničenja makromodela, posebno tamo gdje su prosječne brzine iz VISUM-a nedovoljne za preciznu lokalnu emisijsku analizu.

3.3. Sloj C – Real-time Data Layer

Ovaj sloj obuhvata prikupljanje i obradu stvarnih podataka iz urbanog sistema.

Izvori podataka:

- automatski brojači saobraćaja

- ANPR kamere

- video detekcija i AI klasifikacija vozila

- floating car data

- GPS javnog prevoza

- IoT senzori

- meteorološke stanice

- stanice kvaliteta zraka

Varijable:

- intenzitet saobraćaja

- struktura voznog parka

- stvarne brzine

- travel time

- temperatura

- vlažnost

- brzina vjetra

- PM10, PM2.5, NOx, CO₂

Ovaj sloj služi za:

- verifikaciju simulacijskih rezultata

- ažuriranje modela

- kontinuiranu kalibraciju

- pokretanje alert mehanizama

3.4. Sloj D – Integracioni sloj VISUM–SUMO

Ovo je ključni sloj metodologije.

Integracija se izvodi u dva smjera:

a) VISUM → SUMO

VISUM prenosi prema SUMO-u:

- mrežu

- OD tokove

- matrice potražnje

- scenarije opterećenja

- vremenske profile potražnje

b) SUMO → VISUM / Digital Twin

SUMO vraća:

- detaljne brzine

- kašnjenja

- redove čekanja

- signalne performanse

- lokalne emisijske profile

Na taj način se dobija hibridni model:

- VISUM za gradski/regionalni nivo

- SUMO za kritične lokalne podmreže

4. Metodološki tok rada

Faza 1 – Definisanje sistema i obuhvata

Prvo se definišu:

- prostorni obuhvat

- granice studijskog područja

- zonski sistem

- osnovna saobraćajna mreža

- kritični koridori i raskrsnice za SUMO podsistem

- ciljevi modela

Izlaz:

- konceptualni model sistema

- lista scenarija

- lista ulaznih podataka

Faza 2 – Izrada i validacija VISUM modela

Koraci:

- kodiranje mreže

- unos kapaciteta, broja traka, slobodnih brzina

- definisanje funkcija otpora

- generisanje/distribucija putovanja

- modalna raspodjela

- assignment

- validacija prema brojanjima

Validacija:

- GEH statistika

- poređenje brzina

- screenline provjera

- V/C analiza

- travel time validacija

Izlaz:

- bazni makroskopski model

- bazni scenarij tokova i brzina

Faza 3 – Izrada SUMO modela

Kritični dijelovi mreže izvoze se iz VISUM-a ili kodiraju posebno za SUMO.

Koraci:

- eksport mreže

- konverzija u SUMO format

- dodavanje traka, pravila i signalizacije

- definisanje ruta i demand profila

- kalibracija car-following i lane-changing parametara

- validacija prema lokalnim mjerenjima

Posebno se modeliraju:

- raskrsnice

- prioriteti

- semaforske faze

- autobuske trake

- pješački uticaji

- teretna vozila

Izlaz:

- detaljan operativni model kritičnih zona

Faza 4 – Emisijski model

Emisije se računaju na dva nivoa.

4.1. Makroskopski nivo (VISUM)

Za cijelu mrežu koristi se formula:

El=ql×Ll×EF(vl)

gdje je:

- Elemisija na linku

- qlprotok vozila

- Lldužina linka

- EF(vl)emisijski faktor zavisan od brzine

4.2. Mikroskopski nivo (SUMO)

Za kritične zone koristi se:

Ei,t=f(vi,t,ai,t,typei,fueli,Euroi)

gdje su:

- vi,tbrzina vozila i u vremenu t

- ai,tubrzanje/usporenje

- typeitip vozila

- fueligorivo

- Euroiemisijska klasa

Emisijski izlazi:

- CO₂

- NOx

- PM10

- PM2.5

- CO

- HC

- fuel consumption

- energy use

Logika:

- VISUM daje mrežni pregled

- SUMO daje lokalnu preciznost

5. Digital Twin Engine

Digital Twin engine je centralni modul koji:

- prima real-time podatke

- poredi ih sa modeliranim vrijednostima

- koriguje parametre modela

- pokreće scenarije

- generiše predikcije

Glavna funkcija:

El(t)=ql(t)×Ll×EF(vl(t),fleet(t),meteo(t))

Digital Twin uključuje:

- vremensku komponentu

- promjenu voznog parka

- meteorološke uticaje

- adaptivnu korekciju parametara

6. Kalibracija i rekalibracija

6.1. Inicijalna kalibracija

Radi se na osnovu:

- brojanja

- brzina

- vremena putovanja

- semaforskih ciklusa

- strukture voznog parka

6.2. Operativna rekalibracija

Radi se periodično ili automatski kada se uoče odstupanja između:

- realnih tokova i simuliranih tokova

- realnih brzina i simuliranih brzina

- realnih koncentracija i procijenjenih emisija

Parametri koji se koriguju:

- OD matrice

- VDF parametri

- demand profile

- signalni planovi

- fleet composition

- emisijski faktori

7. AI i prediktivni modul

U naprednoj verziji metodologije uključuje se AI modul.

Funkcije AI modula:

- kratkoročna predikcija brzina i tokova

- predikcija zagušenja

- predikcija emisija po satima

- detekcija anomalija

- preporuka upravljačkih mjera

Mogući algoritmi:

- Random Forest

- XGBoost

- LSTM

- Graph Neural Networks

- anomaly detection models

Ulazi:

- historijski tokovi

- real-time podaci

- meteorološki podaci

- signalni planovi

- događaji i incidenti

8. Scenarijska analiza

Metodologija predviđa evaluaciju više scenarija.

Tipovi scenarija:

- bazni scenarij

- nova saobraćajnica

- zatvaranje koridora

- adaptivna signalizacija

- bus priority

- low emission zone

- restrikcija teretnog saobraćaja

- promjena modal split-a

- elektrifikacija voznog parka

Za svaki scenarij računaju se:

- ukupne emisije

- emisije po zoni

- emisije po koridoru

- vremena putovanja

- prosječne brzine

- dužine redova

- izloženost stanovništva

9. Vizualizacija i dashboard

Rezultati se prikazuju kroz Digital Twin dashboard.

Elementi dashboarda:

- GIS karta emisija

- karta zagušenja

- karta kvaliteta zraka

- KPI panel

- poređenje scenarija

- alarmi

- vremenske serije

Ključni KPI:

- CO₂ po koridoru

- NOx po satu

- PM10 po zoni

- average speed

- delay

- queue length

- VKT

- exposure index

- congestion index

10. Predloženi tehnički stack

Modeliranje:

- PTV Visum

- SUMO

Obrada i integracija:

- Python

- Pandas

- GeoPandas

- NumPy

- FastAPI ili Flask

Baza:

- PostgreSQL/PostGIS

Vizualizacija:

- QGIS

- Power BI

- Grafana

- web GIS dashboard

Razmjena podataka:

- CSV

- XML

- API

- JSON

- TraCI za SUMO

11. Predložena implementaciona procedura

Korak 1

Razviti i validirati bazni VISUM model.

Korak 2

Identifikovati kritične koridore za SUMO.

Korak 3

Izvesti mrežu i demand profile u SUMO.

Korak 4

Kalibrisati SUMO prema realnim mjerenjima.

Korak 5

Povezati emisijski model sa oba nivoa.

Korak 6

Uvesti real-time podatke i bazu.

Korak 7

Razviti dashboard.

Korak 8

Dodati AI predikciju.

Korak 9

Testirati operativne scenarije.

12. Naučni i praktični doprinos metodologije

Ova metodologija daje tri nivoa doprinosa.

Naučni doprinos

- integracija makro i mikro modeliranja

- integracija simulacije i realnih podataka

- dinamička procjena emisija

Stručni doprinos

- podrška SUMP-u

- podrška ITS upravljanju

- podrška ekološkim zonama i politikama

Praktični doprinos

- real-time uvid u stanje mreže

- procjena efekata mjera prije implementacije

- podrška kriznom upravljanju tokom epizoda zagađenja

13. Ograničenja metodologije

Potrebno je jasno navesti i ograničenja:

- zavisnost od kvaliteta ulaznih podataka

- potreba za dobrom kalibracijom oba modela

- ograničena dostupnost real-time podataka u BiH

- složenost integracije VISUM–SUMO

- potreba za redovnim ažuriranjem voznog parka

- neizvjesnosti kod meteorološke disperzije i cold-start efekata

14. Predloženi tekst zaključka za metodološko poglavlje

Predložena metodologija integracije PTV Visum i SUMO modela unutar Digital Twin okvira omogućava višeslojnu analizu urbanog transportnog sistema, u kojoj se strateški makroskopski prikaz mreže dopunjuje operativnom mikroskopskom simulacijom i real-time podacima. Takav pristup omogućava ne samo precizniju procjenu emisija saobraćaja, već i razvoj sistema za kontinuirano praćenje, predikciju i optimizaciju urbanog saobraćaja i okolišnih uticaja. U kontekstu urbanih sredina Bosne i Hercegovine, ova metodologija predstavlja realnu osnovu za razvoj pametnih i održivih sistema upravljanja mobilnošću.

Ako želite, sljedeći korak mogu odmah uraditi u jednom od ova dva formata: akademsko poglavlje 5–7 stranica spremno za Word ili vizualni dijagram toka metodologije VISUM + SUMO + Digital Twin za ubacivanje u rad.

kompletnu metodologiju Digital Twin modela sa SUMO + VISUM integracijom

Naravno. U nastavku je kompletna metodologija Digital Twin modela sa SUMO + VISUM integracijom koju možete direktno ugraditi u doktorski rad, projektnu aplikaciju ili metodološko poglavlje. Oslanja se na logiku vašeg postojećeg VISUM-emisijskog pristupa i proširuje ga u operativni Digital Twin okvir .

Kompletna metodologija Digital Twin modela sa SUMO + VISUM integracijom

1. Osnovni koncept metodologije

Predložena metodologija zasniva se na povezivanju:

- PTV Visum kao makroskopskog strateškog modela

- SUMO kao mikroskopskog/mezoskopskog simulacijskog okruženja

- real-time podataka iz senzora, kamera, GPS/floating car data i mjerenja kvaliteta zraka

- emisijskog modula zasnovanog na HBEFA/COPERT logici

- Digital Twin engine-a za kontinuiranu kalibraciju, predikciju i evaluaciju scenarija

Suština pristupa je da VISUM daje stratešku strukturu sistema, dok SUMO daje operativnu dinamiku tokova, a Digital Twin objedinjuje oba nivoa u jedinstven sistem upravljanja emisijama i mobilnošću.

2. Cilj metodologije

Cilj metodologije je razvoj digitalnog blizanca urbanog transportnog sistema koji omogućava:

- kontinuirano praćenje saobraćajnih tokova

- procjenu emisija po dionicama i zonama

- simulaciju infrastrukturnih i operativnih scenarija

- predikciju zagušenja i emisija

- podršku odlučivanju za SUMP, ITS i Smart City upravljanje

3. Višenivojska arhitektura modela

Metodologija se sastoji od šest međusobno povezanih slojeva.

3.1. Sloj A – Strateški model u PTV Visum

Uloga VISUM-a je da obezbijedi:

- zonski sistem

- OD matrice

- klasifikaciju mreže

- kapacitete

- VDF/BPR funkcije

- bazne scenarije razvoja saobraćaja

VISUM se koristi za:

- četverokoračni model potražnje

- raspodjelu tokova na mrežu

- dobijanje agregiranih indikatora:

  - tokovi po linkovima

  - prosječne brzine

  - V/C odnos

  - vremena putovanja

  - opterećenje mreže

Ovaj sloj daje makroskopsku osnovu Digital Twin sistema.

3.2. Sloj B – Operativni simulacijski model u SUMO

SUMO se koristi za detaljniju simulaciju ponašanja vozila na kritičnim koridorima, raskrsnicama i urbanim podmrežama.

SUMO model obuhvata:

- geometriju mreže

- trake

- semaforske planove

- tipove vozila

- rute

- mikrodinamiku kretanja

- ubrzanja/usporenja

- zadržavanja

- redove čekanja

SUMO daje:

- sekunda-po-sekunda ili minutne profile brzine

- dužine redova

- kašnjenja

- zaustavljanja

- detaljnije emisije po vozilu, linku ili zoni

Na taj način SUMO dopunjuje ograničenja makromodela, posebno tamo gdje su prosječne brzine iz VISUM-a nedovoljne za preciznu lokalnu emisijsku analizu.

3.3. Sloj C – Real-time Data Layer

Ovaj sloj obuhvata prikupljanje i obradu stvarnih podataka iz urbanog sistema.

Izvori podataka:

- automatski brojači saobraćaja

- ANPR kamere

- video detekcija i AI klasifikacija vozila

- floating car data

- GPS javnog prevoza

- IoT senzori

- meteorološke stanice

- stanice kvaliteta zraka

Varijable:

- intenzitet saobraćaja

- struktura voznog parka

- stvarne brzine

- travel time

- temperatura

- vlažnost

- brzina vjetra

- PM10, PM2.5, NOx, CO₂

Ovaj sloj služi za:

- verifikaciju simulacijskih rezultata

- ažuriranje modela

- kontinuiranu kalibraciju

- pokretanje alert mehanizama

3.4. Sloj D – Integracioni sloj VISUM–SUMO

Ovo je ključni sloj metodologije.

Integracija se izvodi u dva smjera:

a) VISUM → SUMO

VISUM prenosi prema SUMO-u:

- mrežu

- OD tokove

- matrice potražnje

- scenarije opterećenja

- vremenske profile potražnje

b) SUMO → VISUM / Digital Twin

SUMO vraća:

- detaljne brzine

- kašnjenja

- redove čekanja

- signalne performanse

- lokalne emisijske profile

Na taj način se dobija hibridni model:

- VISUM za gradski/regionalni nivo

- SUMO za kritične lokalne podmreže

4. Metodološki tok rada

Faza 1 – Definisanje sistema i obuhvata

Prvo se definišu:

- prostorni obuhvat

- granice studijskog područja

- zonski sistem

- osnovna saobraćajna mreža

- kritični koridori i raskrsnice za SUMO podsistem

- ciljevi modela

Izlaz:

- konceptualni model sistema

- lista scenarija

- lista ulaznih podataka

Faza 2 – Izrada i validacija VISUM modela

Koraci:

- kodiranje mreže

- unos kapaciteta, broja traka, slobodnih brzina

- definisanje funkcija otpora

- generisanje/distribucija putovanja

- modalna raspodjela

- assignment

- validacija prema brojanjima

Validacija:

- GEH statistika

- poređenje brzina

- screenline provjera

- V/C analiza

- travel time validacija

Izlaz:

- bazni makroskopski model

- bazni scenarij tokova i brzina

Faza 3 – Izrada SUMO modela

Kritični dijelovi mreže izvoze se iz VISUM-a ili kodiraju posebno za SUMO.

Koraci:

- eksport mreže

- konverzija u SUMO format

- dodavanje traka, pravila i signalizacije

- definisanje ruta i demand profila

- kalibracija car-following i lane-changing parametara

- validacija prema lokalnim mjerenjima

Posebno se modeliraju:

- raskrsnice

- prioriteti

- semaforske faze

- autobuske trake

- pješački uticaji

- teretna vozila

Izlaz:

- detaljan operativni model kritičnih zona

Faza 4 – Emisijski model

Emisije se računaju na dva nivoa.

4.1. Makroskopski nivo (VISUM)

Za cijelu mrežu koristi se formula:

El=ql×Ll×EF(vl)

gdje je:

- Elemisija na linku

- qlprotok vozila

- Lldužina linka

- EF(vl)emisijski faktor zavisan od brzine

4.2. Mikroskopski nivo (SUMO)

Za kritične zone koristi se:

Ei,t=f(vi,t,ai,t,typei,fueli,Euroi)

gdje su:

- vi,tbrzina vozila i u vremenu t

- ai,tubrzanje/usporenje

- typeitip vozila

- fueligorivo

- Euroiemisijska klasa

Emisijski izlazi:

- CO₂

- NOx

- PM10

- PM2.5

- CO

- HC

- fuel consumption

- energy use

Logika:

- VISUM daje mrežni pregled

- SUMO daje lokalnu preciznost

5. Digital Twin Engine

Digital Twin engine je centralni modul koji:

- prima real-time podatke

- poredi ih sa modeliranim vrijednostima

- koriguje parametre modela

- pokreće scenarije

- generiše predikcije

Glavna funkcija:

El(t)=ql(t)×Ll×EF(vl(t),fleet(t),meteo(t))

Digital Twin uključuje:

- vremensku komponentu

- promjenu voznog parka

- meteorološke uticaje

- adaptivnu korekciju parametara

6. Kalibracija i rekalibracija

6.1. Inicijalna kalibracija

Radi se na osnovu:

- brojanja

- brzina

- vremena putovanja

- semaforskih ciklusa

- strukture voznog parka

6.2. Operativna rekalibracija

Radi se periodično ili automatski kada se uoče odstupanja između:

- realnih tokova i simuliranih tokova

- realnih brzina i simuliranih brzina

- realnih koncentracija i procijenjenih emisija

Parametri koji se koriguju:

- OD matrice

- VDF parametri

- demand profile

- signalni planovi

- fleet composition

- emisijski faktori

7. AI i prediktivni modul

U naprednoj verziji metodologije uključuje se AI modul.

Funkcije AI modula:

- kratkoročna predikcija brzina i tokova

- predikcija zagušenja

- predikcija emisija po satima

- detekcija anomalija

- preporuka upravljačkih mjera

Mogući algoritmi:

- Random Forest

- XGBoost

- LSTM

- Graph Neural Networks

- anomaly detection models

Ulazi:

- historijski tokovi

- real-time podaci

- meteorološki podaci

- signalni planovi

- događaji i incidenti

8. Scenarijska analiza

Metodologija predviđa evaluaciju više scenarija.

Tipovi scenarija:

- bazni scenarij

- nova saobraćajnica

- zatvaranje koridora

- adaptivna signalizacija

- bus priority

- low emission zone

- restrikcija teretnog saobraćaja

- promjena modal split-a

- elektrifikacija voznog parka

Za svaki scenarij računaju se:

- ukupne emisije

- emisije po zoni

- emisije po koridoru

- vremena putovanja

- prosječne brzine

- dužine redova

- izloženost stanovništva

9. Vizualizacija i dashboard

Rezultati se prikazuju kroz Digital Twin dashboard.

Elementi dashboarda:

- GIS karta emisija

- karta zagušenja

- karta kvaliteta zraka

- KPI panel

- poređenje scenarija

- alarmi

- vremenske serije

Ključni KPI:

- CO₂ po koridoru

- NOx po satu

- PM10 po zoni

- average speed

- delay

- queue length

- VKT

- exposure index

- congestion index

10. Predloženi tehnički stack

Modeliranje:

- PTV Visum

- SUMO

Obrada i integracija:

- Python

- Pandas

- GeoPandas

- NumPy

- FastAPI ili Flask

Baza:

- PostgreSQL/PostGIS

Vizualizacija:

- QGIS

- Power BI

- Grafana

- web GIS dashboard

Razmjena podataka:

- CSV

- XML

- API

- JSON

- TraCI za SUMO

11. Predložena implementaciona procedura

Korak 1

Razviti i validirati bazni VISUM model.

Korak 2

Identifikovati kritične koridore za SUMO.

Korak 3

Izvesti mrežu i demand profile u SUMO.

Korak 4

Kalibrisati SUMO prema realnim mjerenjima.

Korak 5

Povezati emisijski model sa oba nivoa.

Korak 6

Uvesti real-time podatke i bazu.

Korak 7

Razviti dashboard.

Korak 8

Dodati AI predikciju.

Korak 9

Testirati operativne scenarije.

12. Naučni i praktični doprinos metodologije

Ova metodologija daje tri nivoa doprinosa.

Naučni doprinos

- integracija makro i mikro modeliranja

- integracija simulacije i realnih podataka

- dinamička procjena emisija

Stručni doprinos

- podrška SUMP-u

- podrška ITS upravljanju

- podrška ekološkim zonama i politikama

Praktični doprinos

- real-time uvid u stanje mreže

- procjena efekata mjera prije implementacije

- podrška kriznom upravljanju tokom epizoda zagađenja

13. Ograničenja metodologije

Potrebno je jasno navesti i ograničenja:

- zavisnost od kvaliteta ulaznih podataka

- potreba za dobrom kalibracijom oba modela

- ograničena dostupnost real-time podataka u BiH

- složenost integracije VISUM–SUMO

- potreba za redovnim ažuriranjem voznog parka

- neizvjesnosti kod meteorološke disperzije i cold-start efekata

14. Predloženi tekst zaključka za metodološko poglavlje

Predložena metodologija integracije PTV Visum i SUMO modela unutar Digital Twin okvira omogućava višeslojnu analizu urbanog transportnog sistema, u kojoj se strateški makroskopski prikaz mreže dopunjuje operativnom mikroskopskom simulacijom i real-time podacima. Takav pristup omogućava ne samo precizniju procjenu emisija saobraćaja, već i razvoj sistema za kontinuirano praćenje, predikciju i optimizaciju urbanog saobraćaja i okolišnih uticaja. U kontekstu urbanih sredina Bosne i Hercegovine, ova metodologija predstavlja realnu osnovu za razvoj pametnih i održivih sistema upravljanja mobilnošću.

Ako želite, sljedeći korak mogu odmah uraditi u jednom od ova dva formata: akademsko poglavlje 5–7 stranica spremno za Word ili vizualni dijagram toka metodologije VISUM + SUMO + Digital Twin za ubacivanje u rad.